Lab Routines

General guidelines for all lab areas

PCR area

Electrophoresis area

Immuno and western blot area

Extraction area

Isotope lab

Animal experiments’ areas

Microscope room

General guidelines for all lab areas

- Keep updated manuals in appropriate organizers in the labs

- Label materials with date of preparation or first use so that old things can be discarded

- Use only intended materials such as pipettes, forceps and pens in the work areas

  - Do not move materials between work areas or hoods

  - View every work area as a self-contained unit

  - Do not steal from other areas if something runs out

- Usually there is a person routinely running experiments in each area. When planning to run discuss with this person when you can interrupt the routine work

- Throw away hazardous chemicals in appropriate containers

  - When full, put containers in basement room B3:? and replace with an empty container

  - Remember to label the new hazardous waste container with orange sticker: chemical(s) and date

- Mark on whiteboard whenever something in the lab needs to be ordered or addressed:

  - Write what is needed, the date and your name

  - Make sure to write down things to be ordered well ahead of time

  - Notify the person who is going to place the next order that you have added to the whiteboard list

  - If you are the one to place the next order, check the whiteboard first and mark what you will order with e.g. “ordered and date”

  - When receiving shipments, update the order list

- Autoclave trays should be prepared continuously autoclaved when full

  - Autoclaving can be done either in lab or outside the lab

  - Always use tape to show that autoclaving has been done

  - Put clean autoclaved materials in cupboard above the sink next to the extraction room

PCR area

### General guidelines

- Keep clean from all contaminations that can affect PCR

- Don’t clutter the work areas since several people will use the machines

### Daily routines

- Clean LAF-bench areas with alcohol 70% before starting experiments

- Clean LAF-bench areas after finishing and close the hoods

### Weekly routines (Monday)

- Prepare fresh 0.1% DEPC-treated autoclaved water: seven 250ml bottles –open one per day

- Clean LAF-bench areas and contained instruments with RNA:ase removal solution

Electrophoresis area

### General guidelines

- Discard old EB-gels and EB-containing running buffer in special chemical waste containers

- Make sure to keep work area clean to avoid EB-contamination (for personal safety)

### Daily routines

- Rinse beakers for reuse after preparing agarose gels during the day

- Rinse gel-trays and combs immediately after removing from gel

### Weekly routines

- Change agarose running-buffer (TBE 0.5X) weekly or bi-weekly depending on usage

- Prepare fresh stock solution (TBE 5X) when needed (if you finish it, you prepare new)

Immuno and western blot area

- Use surface covers to absorb spill as needed and clean areas after use

- Store antibodies in appropriate place(s)

- Use Bacitracin 0,01% and NaAzid 0,02% when preparing larger amounts of PBS for storage

Extraction area

### General guidelines

- Use extraction room only for extractions

  - Avoid this room when not performing extractions (may change depending on placing of new equipment)

  - Close the door to keep Trizol inside and contaminants outside

- Use ventilated work area for rotor knife homogenisation of tissues and for work with Trizol

- LAF-bench can be used for procedures where low risk of contamination is needed, i.e. late steps in RNA extraction

- Use only autoclaved pipette tips and tubes

### Daily routines

- Clean work areas with anti-RNAse and 70% alcohol before starting extractions

- Autoclave homogenizator rotor-knife and other homogenisation instruments after use

- Throw all Trizol-waste in appropriate container

Isotope lab

### General guidelines

- Foremost is to minimize all risk of radioactive contamination in the lab

- Wear coat, gloves and dosage meter

- Use protective shields when working with radioactive materials

- Consult knowledgeable person or safety-data sheets before working with unfamiliar isotopes

- Use appropriate (depending on isotope used) containers for all disposables

- Remove containers to B3: ? basement storage room when full and replace with empty containers

### Daily routines when using lab

- Use Geiger counter to check areas before and after working

- Clean all used areas after finishing

- Throw away labelled probes that will not be used

Animal experiments’ areas

### Testing areas

- ???

### Surgery areas

- Always clean and return instruments to proper place in steel cabinet.

Microscope room

- Keep slide tray clean

- Keep objectives clean

- Be kind to the mercury lamp

Happy labbing, Erik